Practical No.02: Arithmetic Logic Unit

CODE:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_unsigned.ALL;

entity ALU is

Port (

A: in STD_LOGIC_VECTOR (3 downto 0);

B: in STD_LOGIC_VECTOR (3 downto 0);

Cin : in STD_LOGIC;

Sel : in STD_LOGIC_VECTOR (3 downto 0);

Y : out STD_LOGIC_VECTOR (3 downto 0));

end ALU;

architecture concurrent of ALU is

begin

Y<= AwhenSel ="0000"else

A+1 whenSel="0001" else

B whenSel ="0010" else

B+1 whenSel= "0011" else

A+B whenSel ="0100" else

A+B+Cin whenSel = "0101" else

A-B whenSel ="0110" else

B-1 whenSel = "0111" else

AorBwhenSel="1000"else

Anor BwhenSel="1001"else

AnandBwhenSel="1010"else

AandBwhenSel="1011"else

not(A) when Sel = "1100" else

Axor BwhenSel ="1101" else

Axnor B whenSel ="1110"else

not(B) ;

end concurrent;

TB Code:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_unsigned.ALL;

entity ALU_tb is-- Port ( );

end ALU_tb;

architecture Behavioral of ALU_tb is

component ALUis

Port ( A : in STD_LOGIC_VECTOR (3 downto 0);

B: in STD_LOGIC_VECTOR (3 downto 0);

Cin : in STD_LOGIC;

Sel : in STD_LOGIC_VECTOR (3 downto 0);

Y : out STD_LOGIC_VECTOR (3 downto 0));

end component;

signal A, B, Y : std_logic_vector (3 downto 0);

signal Cin : std_logic;

signal Sel : std_logic_vector(3 downto 0) := "0000";

begin

u1: ALUport map (A, B, Cin, Sel, Y);

process

begin

A<="1010";

B <="1111";

Cin <= '0';

Sel <= Sel + 1;

wait for 100 ns;

Cin <= '1';

wait for 100 ns;

end process;

end Behavioral;

| Port Names | FPGA Pins |
| --- | --- |
| a3 | R2 |
| a2 | T1 |
| a1 | U1 |
| a0 | W2 |
| b3 | R3 |
| b2 | T2 |
| b1 | T3 |
| b0 | V2 |
| s3 | W16 |
| s2 | V16 |
| s1 | V17 |
| y3 | L1 |
| y2 | P1 |
| y1 | N3 |
| y0 | P3 |