Exp1: Half Adder

Code

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity HA is

Port ( a : in STD_LOGIC; -- Input A

b : in STD_LOGIC; -- Input B

s : out STD_LOGIC; -- Sum output

c : out STD_LOGIC); -- Carry output

end HA;

architecture Behavioral of HA is

begin

s <= a xor b;

c <= a and b;

end Behavioral;

TB Code

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity HA_TB is -- Port ( );

end HA_TB;

architecture Behavioral of HA_TB is

component HA is

Port ( A : in STD_LOGIC;

B : in STD_LOGIC;

S : out STD_LOGIC;

C : out STD_LOGIC);

end component;

signal A,B,S,C:std_logic:='0';

begin

uut: HA port map (A=> A ,B=>B ,S=>S ,C=>C);

process

begin

A<='0';

B<='0';

wait for 10ns;

A<='0';

B<='1';

wait for 10ns;

A<='1';

B<='0';

wait for 10ns;

A<='1';

B<='1';

wait for 10ns;

WAIT;

end process;

end Behavioral;

| Port Names | FPGA Pins |
| --- | --- |
| a | R2 |
| b | T1 |
| s | P1 |
| c | L1 |

Full Adder

Code

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

-- Entity declaration

entity Full_Adder is

Port (

a    : in  STD_LOGIC;

b    : in  STD_LOGIC;

cin  : in  STD_LOGIC;

sum  : out STD_LOGIC;

cout : out STD_LOGIC

);

end Full_Adder;

-- Architecture

architecture Behavioral of Full_Adder is

begin

sum  <= a XOR b XOR cin;

cout <= (a AND b) OR (b AND cin) OR (a AND cin);

end Behavioral;

TB CODE

Library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity FA_test is

end FA_test;

architecture Behavioral of FA_test is

component Full_Adder

Port (

a    : in  STD_LOGIC;

b    : in  STD_LOGIC;

cin  : in  STD_LOGIC;

sum  : out STD_LOGIC;

cout : out STD_LOGIC

);

end component;

signal a, b, cin, sum, cout : STD_LOGIC;

begin

-- Instantiate the Full Adder

uut: Full_Adder port map (

a => a,

b => b,

cin => cin,

sum => sum,

cout => cout

);

-- Test Process

process

begin

-- Test all 8 input combinations

a <= '0'; b <= '0'; cin <= '0'; wait for 100 ns;

a <= '0'; b <= '0'; cin <= '1'; wait for 100 ns;

a <= '0'; b <= '1'; cin <= '0'; wait for 100 ns;

a <= '0'; b <= '1'; cin <= '1'; wait for 100 ns;

a <= '1'; b <= '0'; cin <= '0'; wait for 100 ns;

a <= '1'; b <= '0'; cin <= '1'; wait for 100 ns;

a <= '1'; b <= '1'; cin <= '0'; wait for 100 ns;

a <= '1'; b <= '1'; cin <= '1'; wait for 100 ns;

wait; -- stop simulation

end process;

end Behavioral;

| Port Names | FPGA Pins |
| --- | --- |
| a | R2 |
| b | T1 |
| s | P1 |
| co | L1 |
| ci | U1 |

Full Adder Using Half Adder

OR Gate:

entity OR_gate is

Port ( p : in STD_LOGIC;

q : in STD_LOGIC;

r : out STD_LOGIC);

end OR_gate;

architecture Behavioral of OR_gate is

begin

r<= p or q;

end Behavioral;

Half Adder

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity HA is

Port ( a : in STD_LOGIC; -- Input A

b : in STD_LOGIC; -- Input B

s : out STD_LOGIC; -- Sum output

c : out STD_LOGIC); -- Carry output

end HA;

architecture Behavioral of HA is

begin

s <= a xor b;

c <= a and b;

end Behavioral;

Full Adder:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

-- Entity declaration

entity Full_Adder is

Port (

a    : in  STD_LOGIC;

b    : in  STD_LOGIC;

cin  : in  STD_LOGIC;

sum  : out STD_LOGIC;

cout : out STD_LOGIC

);

end Full_Adder;

-- Architecture

architecture Behavioral of Full_Adder is

begin

sum  <= a XOR b XOR cin;

cout <= (a AND b) OR (b AND cin) OR (a AND cin);

end Behavioral;

FA_TB

---code

| Port Names | FPGA Pins |
| --- | --- |
| a | R2 |
| b | T1 |
| s | P1 |
| co | L1 |
| ci | U1 |

Ripple Carry Adder

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity FA is

Port ( a  : in  STD_LOGIC;

b  : in  STD_LOGIC;

ci : in  STD_LOGIC;

s  : out STD_LOGIC;

co : out STD_LOGIC);

end FA;

architecture Behavioral of FA is

begin

s <= a xor b xor ci;

co <= (a and b) or (ci and (a xor b));

end Behavioral;

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity RCA is

Port ( a  : in  STD_LOGIC_VECTOR (3 downto 0);

b  : in  STD_LOGIC_VECTOR (3 downto 0);

ci : in  STD_LOGIC;

s  : out STD_LOGIC_VECTOR (3 downto 0);

co : out STD_LOGIC);

end RCA;

architecture Structural of RCA is

component FA is

Port ( a  : in  STD_LOGIC;

b  : in  STD_LOGIC;

ci : in  STD_LOGIC;

s  : out STD_LOGIC;

co : out STD_LOGIC);

end component;

signal c1, c2, c3 : STD_LOGIC;

begin

FA0: FA port map(

a  => a(0),

b  => b(0),

ci => ci,

s  => s(0),

co => c1

);

FA1: FA port map(

a  => a(1),

b  => b(1),

ci => c1,

s  => s(1),

co => c2

);

FA2: FA port map(

a  => a(2),

b  => b(2),

ci => c2,

s  => s(2),

co => c3

);

FA3: FA port map(

a  => a(3),

b  => b(3),

ci => c3,

s  => s(3),

co => co

);

end Structural;

| Port Names | FPGA Pins |
| --- | --- |
| a3 | R2 |
| a2 | T1 |
| a1 | U1 |
| a0 | W2 |
| b3 | R3 |
| b2 | T2 |
| b1 | T3 |
| b0 | V2 |
| ci | V17 |
| co | U16 |
| s3 | L1 |
| s2 | P1 |
| s1 | N3 |
| s0 | P3 |