Practical No.06: Keypad Interfacing

Code:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity tb_keypad is

Port ( clk : in STD_LOGIC; -- 100 MHz clock

row : in STD_LOGIC_VECTOR (3 downto 0); -- Rows of keypad (inputs)

col : out STD_LOGIC_VECTOR (3 downto 0); -- Columns of keypad (outputs)

seg : out STD_LOGIC_VECTOR (6 downto 0); -- Seven-segment cathodes

an : out STD_LOGIC_VECTOR (3 downto 0)); -- Seven-segment anodes

end tb_keypad;

architecture Behavioral of tb_keypad is

signal col_sel : std_logic_vector(1 downto 0) := "00";

signal display_digit : std_logic_vector(3 downto 0) := "1111"; -- Latched display

signal counter: unsigned(28 downto 0) := (others => '0');

signal clk_div : std_logic := '0';

begin

an <= "1110"; -- Enable rightmost digit

-- Clock divider process

process(clk)

begin

if rising_edge(clk) then

counter <= counter + 1;

if counter = 19 then -- This value determines the scanning speed.

clk_div <= not clk_div;

counter <= (others => '0');

end if;

end if;

end process;

-- Drive columns based on col_sel

with col_sel select

col <= "0111" when "00",

"1011" when "01",

"1101" when "10",

"1110" when "11",

"1111" when others;

-- Keypad scanning and display latching process

process(clk_div)

begin

if rising_edge(clk_div) then

case col_sel is

when "00" =>

if row(3) = '0' then display_digit <= "0001"; -- 1

elsif row(2) = '0' then display_digit <= "0100"; -- 4

elsif row(1) = '0' then display_digit <= "0111"; -- 7

elsif row(0) = '0' then display_digit <= "0000"; -- 0

end if;

when "01" =>

if row(3) = '0' then display_digit <= "0010"; -- 2

elsif row(2) = '0' then display_digit <= "0101"; -- 5

elsif row(1) = '0' then display_digit <= "1000"; -- 8

elsif row(0) = '0' then display_digit <= "1111"; -- F

end if;

when "10" =>

if row(3) = '0' then display_digit <= "0011"; -- 3

elsif row(2) = '0' then display_digit <= "0110"; -- 6

elsif row(1) = '0' then display_digit <= "1001"; -- 9

elsif row(0) = '0' then display_digit <= "1110"; -- E

end if;

when "11" =>

if row(3) = '0' then display_digit <= "1010"; -- A

elsif row(2) = '0' then display_digit <= "1011"; -- B

elsif row(1) = '0' then display_digit <= "1100"; -- C

elsif row(0) = '0' then display_digit <= "1101"; -- D

end if;

when others =>

null;

end case;

col_sel <= std_logic_vector(unsigned(col_sel) + 1); -- Cycle col_sel to scan all columns

end if;

end process;

-- Seven-segment decoder

process(display_digit)

begin

case display_digit is

when "0000" => seg <= "1000000"; -- 0

when "0001" => seg <= "1111001"; -- 1

when "0010" => seg <= "0100100"; -- 2

when "0011" => seg <= "0110000"; -- 3

when "0100" => seg <= "0011001"; -- 4

when "0101" => seg <= "0010010"; -- 5

when "0110" => seg <= "0000010"; -- 6

when "0111" => seg <= "1111000"; -- 7

when "1000" => seg <= "0000000"; -- 8

when "1001" => seg <= "0010000"; -- 9

when "1010" => seg <= "0001000"; -- A

when "1011" => seg <= "0000011"; -- B

when "1100" => seg <= "1000110"; -- C

when "1101" => seg <= "0100001"; -- D

when "1110" => seg <= "0000110"; -- E

when "1111" => seg <= "0001110"; -- F

when others => seg <= "1111111"; -- Blank

end case;

end process;

end Behavioral;

.xdc

set_property PACKAGE_PIN W4 [get_ports {an[3]}]

set_property PACKAGE_PIN V4 [get_ports {an[2]}]

set_property PACKAGE_PIN U4 [get_ports {an[1]}]

set_property PACKAGE_PIN U2 [get_ports {an[0]}]

set_property PACKAGE_PIN G3 [get_ports {row[3]}]

set_property PACKAGE_PIN H2 [get_ports {row[2]}]

set_property PACKAGE_PIN K2 [get_ports {row[1]}]

set_property PACKAGE_PIN H1 [get_ports {row[0]}]

set_property PACKAGE_PIN G2 [get_ports {col[3]}]

set_property PACKAGE_PIN J2 [get_ports {col[2]}]

set_property PACKAGE_PIN L2 [get_ports {col[1]}]

set_property PACKAGE_PIN J1 [get_ports {col[0]}]

set_property PACKAGE_PIN U7 [get_ports {seg[6]}]

set_property PACKAGE_PIN V5 [get_ports {seg[5]}]

set_property PACKAGE_PIN U5 [get_ports {seg[4]}]

set_property PACKAGE_PIN V8 [get_ports {seg[3]}]

set_property PACKAGE_PIN U8 [get_ports {seg[2]}]

set_property PACKAGE_PIN W6 [get_ports {seg[1]}]

set_property PACKAGE_PIN W7 [get_ports {seg[0]}]

set_property PACKAGE_PIN W5 [get_ports clk]

set_property IOSTANDARD LVCMOS33 [get_ports clk]

set_property IOSTANDARD LVCMOS33 [get_ports {seg[6]}]

set_property IOSTANDARD LVCMOS33 [get_ports {seg[5]}]

set_property IOSTANDARD LVCMOS33 [get_ports {seg[4]}]

set_property IOSTANDARD LVCMOS33 [get_ports {seg[3]}]

set_property IOSTANDARD LVCMOS33 [get_ports {seg[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {seg[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {seg[0]}]

set_property IOSTANDARD LVCMOS33 [get_ports {col[3]}]

set_property IOSTANDARD LVCMOS33 [get_ports {col[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {col[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {col[0]}]

set_property IOSTANDARD LVCMOS33 [get_ports {row[3]}]

set_property IOSTANDARD LVCMOS33 [get_ports {row[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {row[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {row[0]}]

set_property IOSTANDARD LVCMOS33 [get_ports {an[3]}]

set_property IOSTANDARD LVCMOS33 [get_ports {an[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {an[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {an[0]}]