Practical No.03: Universal Shift Regiter

Code

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity usr1 is

Port (

clk : in  STD_LOGIC;

clk_div : inout std_logic;

rst : in  STD_LOGIC;

load: in  STD_LOGIC;

mode : in  STD_LOGIC_VECTOR(1 downto 0);  -- mode selection input

si : in  STD_LOGIC;  -- serial input

pi : in  STD_LOGIC_VECTOR(3 downto 0);  -- parallel input

so : out  STD_LOGIC;  -- serial output

po : out  STD_LOGIC_VECTOR(3 downto 0)  -- parallel output

);

end usr1;

architecture Behavioral of usr1 is

signal shift_reg : STD_LOGIC_VECTOR(3 downto 0);

signal counter: std_logic_vector(26 downto 0):=( others=>'0');

begin

process(clk_div, rst,load)

begin

if rst = '1' then

shift_reg <= (others => '0');

elsif clk_div'event and clk_div= '1' then

case mode is

when "00" =>  -- SISO mode

shift_reg( 3 downto 1) <= shift_reg(2 downto 0);

shift_reg(0)<=si;

so <= shift_reg(3);

when "01" =>  -- SIPO mode

shift_reg( 3 downto 1) <= shift_reg(2 downto 0);

shift_reg(0)<=si;

po <= shift_reg;

when "10" =>  -- PISO mode

if(load='0') then

shift_reg <= pi;

else

shift_reg( 3 downto 1) <= shift_reg(2 downto 0);

end if;

so <= shift_reg(3);

when "11" =>  -- PIPO mode

po<= pi;

when others =>

null;

end case;

end if;

end process;

-- so <= shift_reg(3);

-- po <= shift_reg;

process(clk)

begin

if clk'event and clk = '1' then

if rst = '1' then

counter <= (others => '0');

else

counter <= counter + '1';

end if;

end if;

end process;

clk_div<= counter(26);

end Behavioral;

TB-Code

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity usr1tb is

end usr1tb;

architecture Behavioral of usr1tb is

signal clk,clk_div : STD_LOGIC := '0';

signal rst : STD_LOGIC := '0';

signal load : STD_LOGIC := '0';

signal mode : STD_LOGIC_VECTOR(1 downto 0) := "00";

signal si : STD_LOGIC := '0';

signal pi : STD_LOGIC_VECTOR(3 downto 0) := (others => '0');

signal so : STD_LOGIC;

signal po : STD_LOGIC_VECTOR(3 downto 0);

component usr1

Port (

clk : in STD_LOGIC;

clk_div : inout std_logic;

rst : in STD_LOGIC;

load: in STD_LOGIC;

mode:in STD_LOGIC_VECTOR(1 downto 0);

si : in STD_LOGIC;

pi : in STD_LOGIC_VECTOR(3 downto 0);

so : out STD_LOGIC;

po : out STD_LOGIC_VECTOR(3 downto 0)

);

end component;

begin

uut : usr1

Port Map(

clk => clk,

clk_div => clk_div,

rst => rst,

load => load,

mode=>mode,

si => si,

pi => pi,

so => so,

po =>po

);

clk_process : process

begin

clk <= '0';

wait for 10 ns;

clk <= '1';

wait for 10 ns;

end process;

stim_proc : process

begin-- Reset the UUT

rst <= '1';

wait for 20 ns;

rst <= '0';-- Test SISO mode

mode<="00";

si <= '1';

wait for 200 ns;

si <= '0';

wait for 200 ns;-- Test SIPO mode

mode<="01";

si <= '1';

wait for 100 ns;

si <= '0';

wait for 200 ns;-- Test PISO mode

mode<="10";

load<='0';

pi <= "1110";

wait for 100 ns;

load<='1';

wait for 300 ns;-- Test PIPO mode

mode<="11";

pi <= "1010";

wait for 50 ns;

pi <= "1100";

wait for 50 ns;

wait;

end process;

end Behavioral;

| Port Names | FPGA Pins |
| --- | --- |
| clk | W5 |
| clk_div | L1 |
| rst | U1 |
| load | W2 |
| mode1 | R2 |
| mode0 | T1 |
| si | W15 |
| so | W18 |
| pi3 | W17 |
| pi2 | W16 |
| pi1 | V16 |
| pi0 | V17 |
| po3 | V19 |
| po2 | U19 |
| po1 | E19 |
| po0 | U16 |