Practical No.05: FIFO

Code

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity fifo_correct is

GENERIC

(

ADDRESS_WIDTH: integer:=2;---8 bit

DATA_WIDTH: integer:=4---32 bit

);

port ( clk : in std_logic;

clk_div : inout std_logic;

reset : in std_logic;

enr : in std_logic;--enable read,should be '0' when not in use.

enw: in std_logic;--enable write,should be '0' when not in use.

dataout : out std_logic_vector(DATA_WIDTH-1 downto 0);--output data

datain : in std_logic_vector (DATA_WIDTH-1 downto 0);--input data

empty : out std_logic;--set as '1' when the queue is empty

err : out std_logic;

full : out std_logic--set as '1' when the queue is full

);

end fifo_correct;

architecture Behavioral of fifo_correct is

type memory_type is array (0 to ((2**ADDRESS_WIDTH)-1)) of

std_logic_vector(DATA_WIDTH-1 downto 0);-----distributed------

signal memory : memory_type ;-- :=(others => (others => '0'));--memory for

queue.----

signal readptr,writeptr : std_logic_vector(ADDRESS_WIDTH-1 downto 0);--read

and write pointers.

signal full0 : std_logic;

signal empty0 : std_logic;

signal counter: std_logic_vector(28 downto 0):=( others=>'0');

begin

full <= full0;

empty <= empty0;

fifo0: process(clk_div,reset,datain,enw,enr)

begin

if reset='1' then

readptr <= (others => '0');

writeptr <= (others => '0');

empty0 <='1';

full0<='0';

err<='0';

elsif clk_div'event and clk_div = '1' then

if enw='1' and full0='0' then

memory(conv_integer(writeptr)) <= datain ;

writeptr <= writeptr + '1' ;

if (writeptr + '1' = readptr) then

full0<='1';

empty0<= '0';

else

full0<='0';

empty0<= '1';

end if ;

end if ;

if enr='1' and empty0='0' then

dataout <= memory (conv_integer(readptr));

readptr <= readptr + '1' ;

if (readptr + '1' = writeptr ) then

empty0<='1';

full0<='0';

else

empty0<='0';

full0<='1';

end if ;

end if ;

if (empty0='1' and enr='1') or (full0='1' and enw='1') then

err<='1';

else

err<= '0';

end if ;

end if;

end process;

process(clk)

begin

if clk'event and clk= '1' then

counter<= counter + '1';

end if;

end process;

clk_div<= counter(25);

end Behavioral;

TB Code :

LIBRARY ieee;

USE ieee.std_logic_1164.ALL;

ENTITY fifo_test IS

ENDfifo_test;

ARCHITECTURE behavior OF fifo_test IS-- Component Declaration for the Unit Under Test (UUT)

COMPONENTfifo_correct

PORT(

clk : in std_logic;

clk_div : inout std_logic;

reset : in std_logic;

enr : in std_logic;--enable read,should be '0' when not in use.

enw:in std_logic;--enable write,should be '0' when not in use.

dataout : out std_logic_vector(3 downto 0);--output data

datain : in std_logic_vector (3 downto 0);--input data

empty : out std_logic;--set as '1' when the queue is empty

err : out std_logic;

full : out std_logic--set as '1' when the queue is full

);

ENDCOMPONENT;--Inputs

signal clk : std_logic := '0';

signal reset,clk_div: std_logic;

signal enr : std_logic := '0';

signal enw : std_logic := '0';

signal datain : std_logic_vector(3 downto 0) := (others => '0');--Outputs

signal dataout : std_logic_vector(3 downto 0);

signal empty : std_logic;

signal err : std_logic;

signal full : std_logic;-- Clock period definitions

constant clk_period : time := 10 ns;

BEGIN-- Instantiate the Unit Under Test (UUT)

uut: fifo_correct PORT MAP (

clk => clk,

clk_div=>clk_div,

reset=>reset,

enr => enr,

enw=>enw,

dataout => dataout,

datain => datain,

empty => empty,

err => err,

full => full

);-- Clock process definitions

clk_process :process

begin

clk <= '0';

wait for clk_period/2;

clk <= '1';

wait for clk_period/2;

end process;

reset<='1','0' after 50ns;

enw<= '1', '0' after 200 ns ;

enr<= '0','1' after 200 ns ;--Stimulus process

stim_proc: process

begin

datain<="1010";

wait for 10 ns;

datain<="1111";

wait for 10 ns;

datain<="1001";

wait for 10 ns;

datain<="0001";

wait for 10 ns;

end process;

END behavior;

| Port Names | FPGA Pins |
| --- | --- |
| clk | W5 |
| clk_div | P1 |
| rst | U1 |
| err | N3 |
| full | P1 |
| empty | L1 |
| enw(enable write) | R2 |
| enr(enable read) | T1 |
| datain3 | W17 |
| datain2 | W16 |
| datain1 | V16 |
| datain0 | V17 |
| dataout3 | V19 |
| dataout2 | U19 |
| dataout1 | E19 |
| dataout0 | U16 |